HiveNavigator

Take-Home Technical Exercise

Acoustic & Sensor Analysis of Queenright vs. Queenless Bee Colonies

# Background

HiveNavigator is a project aimed at automated hive monitoring that detects biologically significant colony events — swarming, queen loss, parasite pressure — using a continuous stream of acoustic, vibration, and environmental sensor data. The platform collects audio from microphones inside each hive, vibration sensors, temperature, and humidity throughout the day and night.

This exercise focuses on a controlled queen-removal experiment. Hive 3 had their queens removed from 12 March 2026. Hive 4 has been queenless from 9th till 12th March. You are asked to characterise how the acoustic and vibrational signatures of those hives change before, during, and after that intervention — and to contrast them with the hives that retained their queen throughout.

The literature (Ferrari et al. (2008)) establishes clear acoustic correlates for colony states: swarming drives a sharp upward shift in dominant frequency from the normal 100–300 Hz upwards with peaks of 500 Hz, while queenlessness is associated with changes in the amplitude and spectral texture of the broad 200–600 Hz range. Modulation spectrograms — which capture how spectral energy fluctuates over time — add a second dimension that first order spectral descriptors miss.

Figure 1. Three frequency components of a beehive monitored over time.

# Dataset Specification

## Hives and their labels

The experiment spans roughly 10 days (7 – 16 March 2026). There are 7 hives with bees and 1 dummy ( Hive 11). Two hives (referred to below as the “queenless”) underwent queen removal and introduction at different times.

| Hive | Queen Status | Notes |
| --- | --- | --- |
| Hive 3 | Queenless 12 Mar- till now | Queen removed on 12 Mar |
| Hive 4 | Queenless before 12 Mar, Queen introduced on 12 Mar | Queen was not present till 12 Mar; Queen introduced on 12 Mar |
| Other hives | Queenright (control) | Undisturbed throughout |

## File structure

You will receive a directory with the following layout:

Audio/

hive_dd/   audio/MM/DD/hive_01_YYYYMMDD_HHMMSS.flac  (two files per hour)

Sensors/

Hive_dd

hive_03_accelerometer.csv (one file per day)

hive_03_sensors.csv

## Audio files

| Format | FLAC (lossless) |
| --- | --- |
| Sample rate | 16 000 Hz |
| Channels | Mono |

## Accelerometer file — hive_03_accelerometer.csv

Sampled approximately every 60 seconds. Seven columns:

• timestamp — UTC datetime

• f1, f2, f3 — three main frequencies identified from the Fast Fourier Transform (FFT) during a 5-second sampling interval

• m1, m2, m3 — magnitudes of the corresponding main FFT frequencies (f1, f2, f3)

## Sensor file — hive_03_sensors.csv

Sampled every 10 minutes. Ten columns:

• timestamp : UTC datetime

• sht_t, sht_h : SHT temperature (°C) and relative humidity (%)

• co2 : CO₂ concentration (ppm)

• scd_t, scd_h : SCD temperature (°C) and relative humidity (%)

• accel_x, accel_y, accel_z : accelerometer position/orientation measurements

• rssi : Received Signal Strength Indicator (RSSI), Wi-Fi signal strength in dBm between the device and hub

# The Exercise

Your goal is to build a self-contained analysis that makes the spectral character of each hive's state visible and interpretable — before, during, and after the queen-removal window — and to compare the two queenless hives against the queenright control.

## Part 1 — Feature extraction pipeline

For each audio file, compute interpretable spectral features over short analysis frames (e.g. 1–2 s Hann windows, 50% overlap) and then average them into a single feature vector representing that hour:

### Core spectral features (librosa, UrBAN feature repo)

- Mel-Frequency Cepstral Coefficients (MFCCs) — 13 or 20 coefficients; capture the coarse spectral envelope

- Spectral bands statistics, especially the components shown on figure 1(their strength, regularity, spectral width).

- Spectral centroid — frequency-weighted centre of mass

- Spectral bandwidth and spectral rolloff — spread and upper-energy concentration

- Spectral flatness — ratio of geometric to arithmetic mean of power spectrum; high flatness indicates noise-like activity

- Chroma features — 12-bin pitch-class energy; picks up tonal piping

- Zero-crossing rate — proxy for noisiness / high-frequency content

- Root-mean-square energy — overall loudness envelope

Look for more features to extract here:

https://github.com/MuSAELab/UrBAN/blob/main/code/feature_extraction.py

https://librosa.org/

https://pypi.org/project/essentia/

### Modulation spectrograms

The modulation spectrogram represents how the energy in each frequency band fluctuates over time. Compute it as follows:

- Divide the audio into short-time Fourier transform (STFT) frames to obtain an ordinary spectrogram S(t, f).

- Apply a second FFT along the time axis of each frequency row to obtain the modulation spectrum: M(ω_m, f), where ω_m is the modulation frequency.

- Summarise each hour as the average M across time frames, yielding a 2-D map of acoustic frequency vs. modulation rate.

The modulation rates most informative for bee behaviour are typically 1–30 Hz (wing-beat modulation around 250 Hz carrier, queen and worker bee piping, fanning). Flatten M or extract summary statistics (peak modulation frequency per band, total modulation energy) for use in Part 2.

## Part 2 — Temporal visualisation

Aggregate the per-hour feature vectors over the full recording period and produce a visualisation that shows how each feature evolves over time for all the hives simultaneously. Try to highlight the queen-removal window (12–14 Mar) visually, e.g. by a shaded region or annotation. Compare the two queenless hives against the queenright control side by side. Examine both the broad trend (day-over-day) and intraday patterns (day vs. night activity cycles).

Given the number of dimensions involved — multiple hives × multiple features × many days × multiple states — a static image will quickly become unreadable. We therefore recommend an interactive visualisation built with Plotly Dash, Streamlit or any other tool of choice. At minimum, the interface should allow a user to:

- Select which feature(s) to display from a dropdown or checkbox list.

- Toggle individual hives on or off.

- Zoom into a specific date range.

- Optionally overlay the accelerometer and environmental sensor data from Hive 03 in a secondary panel.

## Part 3 — Comparative analysis

Write a short report (max 2 pages, or a clearly structured section of your notebook) that addresses the following questions:

- Which spectral features best separate the queenless state from the queenright baseline? Quantify this, even informally (e.g. mean ± std in each state).

- Do the two queenless hives show consistent signatures, or do they diverge? What might explain any differences?

- How quickly does the queenless signature appear after queen removal, and how quickly does it dissipate after reintroduction?

- Does the modulation spectrogram reveal patterns that plain MFCCs miss?

- Are there correlating signals in the accelerometer or environmental sensor data (Hive 03) that align with the acoustic changes?

# Implementation Pointers

## Recommended libraries

- librosa (Python) — the standard toolkit for the core spectral features. Key functions: librosa.feature.mfcc, librosa.feature.spectral_centroid, librosa.feature.spectral_bandwidth, librosa.feature.spectral_rolloff, librosa.feature.spectral_flatness, librosa.feature.chroma_stft, librosa.feature.rms.

- soundfile or torchaudio — for reading FLAC files efficiently.

- numpy / scipy.signal — for the second-pass FFT in the modulation spectrogram, bandpass filtering (6th-order Butterworth, 100–2000 Hz), and windowing.

- pandas — for aligning the per-hour feature DataFrame with the sensor and accelerometer CSVs by timestamp.

- Plotly / Plotly Dash or Streamlit — for the interactive dashboard.

- scikit-learn (optional) — PCA or t-SNE to reduce the feature space and visualise clustering by state.

## Skeleton for a single audio file

import librosa, soundfile as sf, numpy as np

from scipy.signal import butter, sosfilt

y, sr = sf.read('hive_01_2026-03-12_10-00.flac')  # sr == 16000

# Bandpass 100–2000 Hz – experiment with different settings

sos = butter(6, [100, 2000], btype='band', fs=sr, output='sos')

y = sosfilt(sos, y)

frame_len, hop = 16000, 8000  # 1 s frames, 50% overlap

mfccs  = librosa.feature.mfcc(y=y, sr=sr, n_mfcc=13,

n_fft=frame_len, hop_length=hop)

cent   = librosa.feature.spectral_centroid(y=y, sr=sr,

n_fft=frame_len, hop_length=hop)

#TODO try more features

# Modulation spectrogram: FFT along time axis per frequency bin

S = np.abs(librosa.stft(y, n_fft=frame_len, hop_length=hop))

#this is a very crude way of compressing the four dimensional data that is in a modulation spectrogram so you can do other things

mod_spec = np.abs(np.fft.rfft(S, axis=1))  # shape: (freq_bins, mod_freqs)

# Average over half an hour

feature_vec = np.concatenate([mfccs.mean(axis=1),

cent.mean(axis=1),

mod_spec.mean(axis=1)])

# Room for Exploration

We ask you to experiment and explore the parameters of your data preprocessing pipeline, but not to venture into non-interpretable territory of the neural embeddings for now. You could explore different audio features – gammatone filterbanks, spectral flux. Use sensor data in combination with audio, try simple ML, try anomaly detection.

# Deliverables

Please submit your solution, which could contain the following components:

- A Jupyter notebook (or equivalent Python scripts) with your feature extraction pipeline and all figures from end to end.

- Your dashboard application hosted on a free server.

- The short report on your findings.

We expect this exercise to take 6–10 hours for a well-prepared candidate. If you find that it takes you significantly longer, please narrow the scope and do an in-depth analysis on only one of the aspects of your choice.

Good luck!